Emtron Technologies Pvt. Ltd. 
Flat No-101, B3 Wing, 1st Floor, Divyam Hights,     Gilbert Hill, Shreenath Nagar, Andheri –West,           Mumbai-58 
+91-8080181911                                                                    E-mail: emtron.tech@gmail.com, www.emtrontech.in

Microchip 18F4550 Interface, Signal conditioning, USB, USB-RS-232, 16x2 LCD Interface

This development board centred on PIC18F4550 M 8-bit RISC micro controllers from Microchip.  This controller has 32K flash and 2K RAM. In built 10-bit, up to 13-channel Analog-to-Digital Converter , Four Timer modules (Timer0 to Timer3), Three External Interrupts, Four Crystal modes, including High Precision PLL, Supports up to 32 Endpoints (16 bidirectional), Interface for Off-Chip USB Transceiver.

This module of micro controllers is extensively used for embedded and real-time applications in industry. This board is designed to be a general-purpose development board for Single Chip MCU applications that may be used as an instructional learning aid and also as a development tool in R&D labs in industries.

Board Features:

PIC18F4550-based board developed (TQFP-44 package)                                                             USB powered and programmable Interface.                                                                               Standard +5v Regulated Power Supply.                                                                                        Serial Port Interface through FT232 UART-USB converter .                                                         Large number of on-board I/Os.                                                                                                      Connectors are made available for the interfacing of standard I/O such as switches/LEDs and LCD.                                                                                                                                                      Compatible with standard Microchip Technology software tools (MPLAB IDE and HID Boot loader V2.6a).                                                                                                                                          PIC Kit3-Debugger Interface.                                                                                                                I/O Connectors are compatible to standard interface like Sensors, Opt coupler , 4x1 KBD, Motors etc.

Circuit Diagram:

PCB Design:

PIC 18F4550 Installation Guide

Bootloader is required to make USB enable (programming it using PICKIT3 Programmer through Jtag Port)

- Install MPLAB

- Install c18 compiler (mplabc18-v3.46-windows-lite-installer.exe)

- Install from the folder NET/ NETCFS Driver

- Start MPLAB

- Project->Projectwizard->select device (PIC 18F4550)->select language tool set-> select  mpasmwin, mplink,mccc18,mplib

provide path of all four like C:\Program Files\Microchip\mplabc18\v3.46\bin\mcc18.exe

- Create example folder

- add rm184550.lkr file to the project file (use rm184550lkr while compiling the programs to avoid boot loader overwriting

- add source file (.c)

- locate header files: Project -> built options -> project->

- show directories for -> include search path -> New : C:\Program Files\Microchip\mplabc18\v3.46\h

- Library search path path -> New : C:\Program Files\Microchip\mplabc18\v3.46\lib

- Linker search path->  C:\Program Files\Microchip\mplabc18\v3.46\bin\LKR

- configure -> select device->PIC18F4550

- Debugger -> select tool->  MPLABSIM (for simulations)

- Build the file.

- use HID-BootLoader for programming

- For Programming Press fist BOOT- then RESET release RESET &  then BOOT switches

- In case the board is not detected verify : In Windows -> Control Panel -> Device Manager->

16:  Open Hex File -> select .HEX file -> Program/Verify-> Reset Device

Repeat this step every time.

Reference:

http://www.microchip.com/stellent/idcplg?IdcService=SS_GET_PAGE&nodeId=1406&dDocName=en010014

For Existing Project:

- to modify device: Configure-> Select Device

PICKIT3: Start MPLAB IDE: Directly load Hex File

- Programmer-> Select PICKIT3->Debugger-> Connect it

- File-> import-> LED.Hex file

- Programmer-> Program

PICKIT3: Start MPLAB IDE : HID bootloader

- Programmer-> Select PICKIT3->Debugger-> Connect it

- File-> import-> USB Device - HID - HID Bootloader - C18 - PIC18F4550.Hex file

- Programmer-> Program

To automatically load Program from Pickit for mass Programming: repeat above steps except step 3

3.Automatic Programmer-> Settings-> Programmer to go -> Send Image in Memory (Select)

For PIC & MPLAB videos:

-------------------------------------

https://www.youtube.com/watch?v=aMK8zkWn7I8&list=PL0F54805E3165D763

Example Program: LED Blink

#include <p18f4550.h>

/*The following lines of code perform interrupt vector relocation to work with the USB bootloader. These must be

used with every application program to run as a USB application.*/

extern void _startup (void);

#pragma code _RESET_INTERRUPT_VECTOR = 0x1000

void _reset (void)

{

_asm goto _startup _endasm

}

#pragma code

#pragma code _HIGH_INTERRUPT_VECTOR = 0x1008

void high_ISR (void)

{

}

#pragma code

#pragma code _LOW_INTERRUPT_VECTOR = 0x1018

void low_ISR (void)

{

}

#pragma code

/*End of interrupt vector relocation*/

/*Start of main program*/

void myMsDelay (unsigned int time)

{

unsigned int i, j;

for (i = 0; i < time; i++)

for (j = 0; j < 710; j++);/*Calibrated for a 1 ms delay in MPLAB*/

}

void main()

{

TRISA = 0;

while(1)

{

LATA = 0x30;/*Toggling Port A.4, PORTA.5 pins*/

myMsDelay(1000);

LATA = 0x00;

myMsDelay(1000);

}

}

Example Program: Mixed “C” and Assembly Program

//MPLAB C18 Microchip Inline Assembly

// Page 18 (51288a.pdf)

#include <p18f4550.h>

/*The following lines of code perform interrupt vector relocation to work with the USB bootloader. These must be

used with every application program to run as a USB application.*/

extern void _startup (void);

extern void asm_delay (void);

#pragma code _RESET_INTERRUPT_VECTOR = 0x1000

void _reset (void)

{

_asm goto _startup _endasm

}

#pragma code

#pragma code _HIGH_INTERRUPT_VECTOR = 0x1008

void high_ISR (void)

{

}

#pragma code

#pragma code _LOW_INTERRUPT_VECTOR = 0x1018

void low_ISR (void)

{

}

#pragma code

/*End of interrupt vector relocation*/

/*Start of main program*/

void main()

{

unsigned int count =0000;

_asm

/* User assembly code */

CLRF  LATA, ACCESS

CLRF  TRISA, ACCESS

again:

MOVLW 0xFF

MOVWF LATA, ACCESS

CALL delay, BANKED

MOVLW 0x00

MOVWF LATA, ACCESS

CALL delay, BANKED

_endasm

_asm

delay:

MOVWF count, 0

loop1:

DECFSZ count, 1, 0

GOTO loop1

RETURN BANKED

_endasm

while(1)

{}

}

Example Program: Assembly Program

#include<p18f4450.inc>

CONFIG WDT=OFF; disable watchdog timer

CONFIG MCLRE = ON; MCLEAR Pin on

CONFIG DEBUG = ON; Enable Debug Mode

CONFIG LVP = OFF; Low-Voltage programming disabled (necessary for debugging)

CONFIG FOSC = INTOSCIO_EC;Internal oscillator, port function on RA6

org 0; start code at 0

Delay1 res 2 ;reserve 2 byte for the variable Delay1

Start:

CLRF LATD

CLRF TRISD

CLRF Delay1

MainLoop:

movlw   0xff	;

movwf   LATD

Delay11:

DECFSZ Delay1,1 ;Decrement Delay1 by 1, skip next instruction if Delay1 is 0

GOTO Delay11

movlw   0x00	;

movwf   LATD

Delay22:

DECFSZ Delay1,1 ;Decrement Delay1 by 1, skip next instruction if Delay1 is 0

GOTO Delay22

GOTO MainLoop

end